|  | Simulation of Assem4 Date: Sunday, January 12, 2025 Designer: Solidworks Study name: Static 4 Analysis type: Static. Table of Contents Description 1 Material Properties 2 Connector Definitions 4 Mesh information 8 Study Results 9 Conclusion 12. |
| --- | --- |
| # Description No Data |  |

# Material Properties Model Reference; Properties; Components. Name:; 1023 Carbon Steel Sheet (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 2.82685e+08 N/m^2. Tensile strength:; 4.25e+08 N/m^2. Elastic modulus:; 2.05e+11 N/m^2. Poisson's ratio:; 0.29. Mass density:; 7,858 kg/m^3. Shear modulus:; 8e+10 N/m^2. Thermal expansion coefficient:; 1.2e-05 /Kelvin.; SolidBody 1(Cut-Extrude5)(Back-1), SolidBody 1(Cut-Extrude8)(Iteration4_chassis-1), SolidBody 2(Mirror50)(Iteration4_chassis-1), SolidBody 3(Fillet33)(Iteration4_chassis-1), SolidBody 4(Boss-Extrude33)(Iteration4_chassis-1), SolidBody 1(Motor, BLDC, Hoverboard Wheel, 6.5in.stp<1>)(Motor, BLDC, Hoverboard Wheel, 6.5in.stp-1), SolidBody 1(Motor, BLDC, Hoverboard Wheel, 6.5in.stp<1>)(Motor, BLDC, Hoverboard Wheel, 6.5in.stp-2), SolidBody 1(Chamfer4)(RS.177-4516-3000N-1/RS.177-4516-bottom-1), SolidBody 1(Chamfer2)(RS.177-4516-3000N-1/RS.177-4516-bottom-mount-1), SolidBody 1(Chamfer4)(RS.177-4516-3000N-2/RS.177-4516-bottom-1), SolidBody 1(Chamfer2)(RS.177-4516-3000N-2/RS.177-4516-bottom-mount-1), SolidBody 1(Fillet5)(butt_cushion_v2-1), SolidBody 1(HeadChamfer)(heavy hex bolt_am-2), SolidBody 1(HeadChamfer)(heavy hex bolt_am-3). Curve Data:N/A. Name:; Cast Alloy Steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 2.41275e+08 N/m^2. Tensile strength:; 4.48083e+08 N/m^2. Elastic modulus:; 1.9e+11 N/m^2. Poisson's ratio:; 0.26. Mass density:; 7,300 kg/m^3. Shear modulus:; 7.8e+10 N/m^2. Thermal expansion coefficient:; 1.5e-05 /Kelvin.; SolidBody 1(Fillet2)(Hub Motor Shaft Bracket-1), SolidBody 1(Fillet2)(Hub Motor Shaft Bracket-2). Curve Data:N/A. Name:; 1060 Alloy. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 2.75742e+07 N/m^2. Tensile strength:; 6.89356e+07 N/m^2. Elastic modulus:; 6.9e+10 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2,700 kg/m^3. Shear modulus:; 2.7e+10 N/m^2. Thermal expansion coefficient:; 2.4e-05 /Kelvin.; SolidBody 1(Cut-Extrude1)(RS.177-4516-3000N-1/RS.177-4516-shaft-1), SolidBody 1(Boss-Extrude2)(RS.177-4516-3000N-1/RS.177-4516-top-mount-1), SolidBody 1(Cut-Extrude1)(RS.177-4516-3000N-2/RS.177-4516-shaft-1), SolidBody 1(Boss-Extrude2)(RS.177-4516-3000N-2/RS.177-4516-top-mount-1). Curve Data:N/A.

# Connector Definitions Connector Name; Connector Details; Connector Image. Rigid Connector-1; Entities:; 4 face(s). Type:; Rigid.; Rigid Connector-1.. Rigid Connector-2; Entities:; 6 face(s). Type:; Rigid.; Rigid Connector-2.. Rigid Connector-3; Entities:; 2 face(s). Type:; Rigid.; Rigid Connector-3.. Rigid Connector-4; Entities:; 2 face(s). Type:; Rigid.; Rigid Connector-4.. Pin/Bolt/Bearing Connector Model Reference; Connector Details; Strength Details. Pin Connector-1; Entities:; 3 face(s). Type:; Pin. With retaining ring (No translation):; Yes. With key (No rotation):; No. Connection Type:; Distributed. Units:; SI. Rotational stiffness value:; 0.; No Data. Connector Forces Joint 1 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); -0; -0; -2.2874; -2.2874. Shear Force (N); -1.9738; -5.4127; 0; 5.7614. Torque (N.m); 0; 0; 4.7885e-14; 4.7885e-14. Bending moment (N.m); 0.016524; 0.056506; 0; 0.058873.. Connector Forces Joint 2 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); -0; -0; -8.1281; -8.1281. Shear Force (N); 5.2673; 1.7594; 0; 5.5534. Torque (N.m); 0; 0; 4.8463e-14; 4.8463e-14. Bending moment (N.m); -0.011985; 0.090652; 0; 0.091441.. Connector Forces Joint 3 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); 0; 0; 8.1281; 8.1281. Shear Force (N); -5.2673; -1.7594; 0; 5.5534. Torque (N.m); 0; 0; 4.4326e-14; 4.4326e-14. Bending moment (N.m); -0.0048462; -0.040265; 0; 0.040556.. Pin Connector-2; Entities:; 3 face(s). Type:; Pin. With retaining ring (No translation):; Yes. With key (No rotation):; No. Connection Type:; Distributed. Units:; SI. Rotational stiffness value:; 0.; No Data. Connector Forces Joint 1 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); -0; -0; -6.2324; -6.2324. Shear Force (N); -4.1864; -1.2741; 0; 4.376. Torque (N.m); 0; 0; 5.1269e-14; 5.1269e-14. Bending moment (N.m); 0.0047929; 0.020975; 0; 0.021516.. Connector Forces Joint 2 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); -0; -0; -6.2324; -6.2324. Shear Force (N); -4.1864; -1.2741; 0; 4.376. Torque (N.m); 0; 0; 5.4082e-14; 5.4082e-14. Bending moment (N.m); 0.0074695; -0.061265; 0; 0.061719.. Connector Forces Joint 3 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); 0; 0; 1.308; 1.308. Shear Force (N); -1.0284; -4.0258; 0; 4.1551. Torque (N.m); 0; 0; 5.3767e-14; 5.3767e-14. Bending moment (N.m); -0.012048; -0.037041; 0; 0.038951.. Pin Connector-3; Entities:; 3 face(s). Type:; Pin. With retaining ring (No translation):; Yes. With key (No rotation):; No. Connection Type:; Distributed. Units:; SI. Rotational stiffness value:; 0.; No Data. Connector Forces Joint 1 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); -0; -0; -2.388; -2.388. Shear Force (N); 8.1801; 5.7623; 0; 10.006. Torque (N.m); 0; 0; 9.789e-14; 9.789e-14. Bending moment (N.m); -0.0019879; -0.018697; 0; 0.018803.. Connector Forces Joint 2 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); -0; -0; -2.388; -2.388. Shear Force (N); 8.1801; 5.7623; 0; 10.006. Torque (N.m); 0; 0; 9.3749e-14; 9.3749e-14. Bending moment (N.m); -0.054848; 0.09938; 0; 0.11351.. Connector Forces Joint 3 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); 0; 0; 2.7904; 2.7904. Shear Force (N); 5.8166; 5.1596; 0; 7.7752. Torque (N.m); 0; 0; 8.5747e-14; 8.5747e-14. Bending moment (N.m); -0.0023396; 0.021277; 0; 0.021406.. Pin Connector-4; Entities:; 3 face(s). Type:; Pin. With retaining ring (No translation):; Yes. With key (No rotation):; No. Connection Type:; Distributed. Units:; SI. Rotational stiffness value:; 0.; No Data. Connector Forces Joint 1 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); -0; -0; -2.2071; -2.2071. Shear Force (N); 4.6516; 3.8315; 0; 6.0264. Torque (N.m); 0; 0; 8.5446e-14; 8.5446e-14. Bending moment (N.m); 0.0011299; -0.017121; 0; 0.017158.. Connector Forces Joint 2 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); -0; -0; -1.7826; -1.7826. Shear Force (N); -6.314; -4.2446; 0; 7.6081. Torque (N.m); 0; 0; 9.1797e-14; 9.1797e-14. Bending moment (N.m); 0.039748; -0.076997; 0; 0.086651.. Connector Forces Joint 3 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); 0; 0; 1.7826; 1.7826. Shear Force (N); 6.314; 4.2446; 0; 7.6081. Torque (N.m); 0; 0; 9.4756e-14; 9.4756e-14. Bending moment (N.m); 0.001432; 0.015739; 0; 0.015804.. Pin Connector-5; Entities:; 3 face(s). Type:; Pin. With retaining ring (No translation):; Yes. With key (No rotation):; No. Connection Type:; Distributed. Units:; SI. Rotational stiffness value:; 0.; No Data. Connector Forces Joint 1 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); 0; 0; 11.762; 11.762. Shear Force (N); 9.8664; 59.092; 0; 59.91. Torque (N.m); 0; 0; 1.6463e-13; 1.6463e-13. Bending moment (N.m); -0.84367; 0.14284; 0; 0.85567.. Connector Forces Joint 2 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); 0; 0; 20.411; 20.411. Shear Force (N); -51.489; -255.28; 0; 260.42. Torque (N.m); 0; 0; 1.689e-13; 1.689e-13. Bending moment (N.m); 3.0456; -0.6975; 0; 3.1244.. Connector Forces Joint 3 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); -0; -0; -20.411; -20.411. Shear Force (N); 51.489; 255.28; 0; 260.42. Torque (N.m); 0; 0; 1.3824e-13; 1.3824e-13. Bending moment (N.m); 1.8126; -0.28236; 0; 1.8344.. Pin Connector-6; Entities:; 3 face(s). Type:; Pin. With retaining ring (No translation):; Yes. With key (No rotation):; No. Connection Type:; Distributed. Units:; SI. Rotational stiffness value:; 0.; No Data. Connector Forces Joint 1 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); 0; 0; -12.016; 12.016. Shear Force (N); 7.8852; 61.841; 0; 62.342. Torque (N.m); -0; -0; 1.6754e-13; -1.6754e-13. Bending moment (N.m); 0.88435; -0.11131; 0; 0.89133.. Connector Forces Joint 2 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); 0; 0; -20.584; 20.584. Shear Force (N); -55.616; -262.09; 0; 267.92. Torque (N.m); -0; -0; 1.7168e-13; -1.7168e-13. Bending moment (N.m); -3.1296; 0.74062; 0; 3.216.. Connector Forces Joint 3 Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (N); -0; -0; 20.584; -20.584. Shear Force (N); 55.616; 262.09; 0; 267.92. Torque (N.m); -0; -0; 1.4108e-13; -1.4108e-13. Bending moment (N.m); -1.8548; 0.3171; 0; 1.8817..

# Mesh information Mesh type; Mixed Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Jacobian check for shell; Off. Maximum element size; 99.1703 mm. Minimum element size; 4.95852 mm. Mesh Quality; High.

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 0.000e+00N/m^2 Node: 9285; 4.447e+07N/m^2 Node: 409851. Assem4-Static 4-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 20338; 3.567e-01mm Node: 234488. Assem4-Static 4-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 0.000e+00 Element: 4608; 1.118e-04 Element: 172373. Assem4-Static 4-Strain-Strain1. Name; Type; Min; Max. Factor of Safety1; Automatic; 6.356e+00 Node: 409851; 1.000e+16 Node: 16854. Assem4-Static 4-Factor of Safety-Factor of Safety1.

# Conclusion